Danmarks Tekniske Universitet

Department of Civil Engineering

# Multi-Sample Comparison Report

Hydraulic Conductivity Analysis - 15 Samples

Project: DTU Sustiain

Date: June 29, 2026

# Multi-Sample Comparison Report

| Project | DTU Sustiain |
| --- | --- |
| Report Date | June 29, 2026 at 13:31 |

Number of Samples: 15

Temperature: 20 °C

Porosity: Varies by sample (0.26 to 0.435)

## Executive Summary

Comparison Analysis: This report compares 15 soil samples based on grain size distribution and hydraulic conductivity estimates.

Key Findings:

Highest permeability: Sample_1_Odong (9.54e-04 m/s)

Lowest permeability: Example_2_Case_1_Vukovic_15C (1.14e-06 m/s)

Permeability range: 837.4x difference

## Methodology

### Comparative Analysis Approach

This comparison report presents a side-by-side analysis of multiple soil samples to identify patterns, variations, and relationships between grain size characteristics and hydraulic conductivity estimates.

### Analysis Components

Grain Size Comparison: Overlapping distribution curves allow visual assessment of particle size variations between samples.

K-Value Comparison: Box plots and statistical summaries reveal the range and reliability of hydraulic conductivity estimates across samples.

Method Reliability: A reliability matrix shows which empirical methods are applicable for each sample, helping identify the most suitable estimation approaches.

## Results & Analysis

### Sample Overview

| Sample | Temp (?C) | Porosity | D10 (mm) | D50 (mm) | D60 (mm) | Cu |
| --- | --- | --- | --- | --- | --- | --- |
| Example_1_Case_1_Vukovic | 20.00 | 0.384 | 0.150 | 0.485 | 0.550 | 3.67 |
| Example_1_Case_2_Vukovic_15C | 20.00 | 0.384 | 0.150 | 0.480 | 0.550 | 3.67 |
| Example_2_Case_1_Vukovic_15C | 20.00 | 0.26 | 0.014 | 0.210 | 0.300 | 21.43 |
| Sample_1_Odong | 20.00 | 0.346 | 0.341 | 1.181 | 1.886 | 5.53 |
| Thomson_SERDP_Borden_Sand | 20.00 | 0.407 | 0.070 | 0.180 | 0.195 | 2.79 |
| NBAL-01 [English] | 20.00 | 0.435 | 0.171 | 0.295 | 0.319 | 1.86 |
| NBAL-02 [English] | 20.00 | 0.399 | 0.255 | 0.655 | 0.785 | 3.08 |
| NBAL-03 [English] | 20.00 | 0.405 | 0.122 | 0.305 | 0.350 | 2.86 |
| NBAL-04 [English] | 20.00 | 0.35 | N/A | 0.173 | 0.205 | N/A |
| NBAL-05 [English] | 20.00 | 0.419 | 0.098 | 0.192 | 0.233 | 2.38 |
| NBAL-06 [English] | 20.00 | 0.421 | 0.257 | 0.526 | 0.592 | 2.30 |
| NBAL-07 [English] | 20.00 | 0.425 | 0.219 | 0.425 | 0.473 | 2.16 |
| NBAL-08 [English] | 20.00 | 0.423 | 0.125 | 0.244 | 0.281 | 2.25 |
| NBAL-09 [English] | 20.00 | 0.432 | 0.164 | 0.297 | 0.325 | 1.97 |
| NBAL-10 [English] | 20.00 | 0.394 | 0.178 | 0.516 | 0.583 | 3.27 |

| Sample | Soil type | K geometric mean (m/s) |
| --- | --- | --- |
| Example_1_Case_1_Vukovic | Poorly-graded sand | 2.73e-04 |
| Example_1_Case_2_Vukovic_15C | Poorly-graded sand | 2.74e-04 |
| Example_2_Case_1_Vukovic_15C | Well-graded sand | 1.14e-06 |
| Sample_1_Odong | Poorly-graded sand | 9.54e-04 |
| Thomson_SERDP_Borden_Sand | Poorly-graded sand | 5.01e-05 |
| NBAL-01 [English] | Poorly-graded sand | 3.64e-04 |
| NBAL-02 [English] | Poorly-graded sand | 7.45e-04 |
| NBAL-03 [English] | Poorly-graded sand | 1.94e-04 |
| NBAL-04 [English] | Sand | 8.50e-05 |
| NBAL-05 [English] | Poorly-graded sand | 1.21e-04 |
| NBAL-06 [English] | Poorly-graded sand | 8.28e-04 |
| NBAL-07 [English] | Poorly-graded sand | 6.05e-04 |
| NBAL-08 [English] | Poorly-graded sand | 1.94e-04 |
| NBAL-09 [English] | Poorly-graded sand | 3.38e-04 |
| NBAL-10 [English] | Poorly-graded sand | 4.18e-04 |

### Grain Parameters Comparison

Large table moved to companion Excel appendix: Grain Parameters Comparison.

### K-Value Aggregate Summary

| Scope | Datasets | Geometric mean (m/s) | Arithmetic mean (m/s) | Median (m/s) | ln(K) std dev |
| --- | --- | --- | --- | --- | --- |
| Overall | 15 | 2.49e-04 | 4.88e-04 | 3.02e-04 | 1.525 |
| test | 3 | 3.98e-04 | 6.39e-04 | 3.44e-04 | 0.931 |
| test2 | 2 | 1.15e-05 | 4.10e-05 | 3.41e-05 | 2.261 |
| test3 | 7 | 3.44e-04 | 5.53e-04 | 3.99e-04 | 1.060 |
| test4 | 3 | 3.01e-04 | 3.97e-04 | 2.73e-04 | 0.716 |

| Scope | Included K cells | Warning cells | Permeability class |
| --- | --- | --- | --- |
| Overall | 188 / 240 | 52 | High (Clean Sand) |
| test | 40 / 48 | 8 | High (Clean Sand) |
| test2 | 18 / 32 | 14 | Moderate (Fine Sand) |
| test3 | 89 / 112 | 23 | High (Clean Sand) |
| test4 | 41 / 48 | 7 | High (Clean Sand) |

### K-Value Calculations by Dataset and Method

Large table moved to companion Excel appendix: K-Value Calculations by Dataset and Method.

### Permeability Classification Summary

| Sample Name | K geometric mean (m/s) | Classification |
| --- | --- | --- |
| Example_1_Case_1_Vukovic | 2.73e-04 | High (Clean Sand) |
| Example_1_Case_2_Vukovic_15C | 2.74e-04 | High (Clean Sand) |
| Example_2_Case_1_Vukovic_15C | 1.14e-06 | Low (Silt) |
| Sample_1_Odong | 9.54e-04 | High (Clean Sand) |
| Thomson_SERDP_Borden_Sand | 5.01e-05 | Moderate (Fine Sand) |
| NBAL-01 [English] | 3.64e-04 | High (Clean Sand) |
| NBAL-02 [English] | 7.45e-04 | High (Clean Sand) |
| NBAL-03 [English] | 1.94e-04 | High (Clean Sand) |
| NBAL-04 [English] | 8.50e-05 | Moderate (Fine Sand) |
| NBAL-05 [English] | 1.21e-04 | High (Clean Sand) |
| NBAL-06 [English] | 8.28e-04 | High (Clean Sand) |
| NBAL-07 [English] | 6.05e-04 | High (Clean Sand) |
| NBAL-08 [English] | 1.94e-04 | High (Clean Sand) |
| NBAL-09 [English] | 3.38e-04 | High (Clean Sand) |
| NBAL-10 [English] | 4.18e-04 | High (Clean Sand) |

## Grain Size Distribution Comparison

## Hydraulic Conductivity by Method

## Hydraulic Conductivity Distribution

## Interpretation & Discussion

### Comparative Analysis

Permeability Characteristics:

The highest permeability sample is Sample_1_Odong with K = 9.54e-04 m/s, classified as High (Clean Sand).

The lowest permeability sample is Example_2_Case_1_Vukovic_15C with K = 1.14e-06 m/s, classified as Low (Silt).

The 837.4-fold difference in permeability reflects the variability in grain size distribution among the samples.

Statistical Overview:

Average sample K geometric mean: 3.63e-04 m/s

Standard deviation: 2.84e-04 m/s

Coefficient of variation: 78.3%